Anticipated Project Impacts and Mitigation Measures

For each valued component, the potential impact, mitigation measure and conclusion (residual effects and significance) are summarized in the table below.

The mitigation measures in the table below include the mitigation measures proposed by Hydro-Québec, the mitigation measures of NRCan, as well as those proposed by Environment and Climate Change Canada, which was consulted under section 85 of the Impact Assessment Act.

| Valued Component | Activity | Impact | Mitigation measures | Residual effect |
| --- | --- | --- | --- | --- |
| Health condition | Noise from machinery and vibrations during excavation | Exposure to noise and the effects of vibration | - Work only during the day, not at night | Not significant |
| Environmental conditions | Accidental spill of petroleum products during construction of the project | In the event of a spill, release of a polluting substance on or in the soil, water or air | - Use only machinery that is clean and in good repair - For each piece of machinery on site, have an oil product recovery kit according to Hydro-Québec criteria - Remind all on-site stakeholders of the alert structure in place in case of spill at the start-up meeting - In the event of a spill, follow the procedure and recover all contaminated soil and water | Not significant |
| Environmental conditions | Excavation activities and movement of machinery | Suspended solids emissions from excavation activities and machinery movement | - Installation of a sediment barrier along the shore to limit the dispersion of sediment in the watercourses present. - The soils stored on the site will be covered to prevent wind dispersion. - Refurbishment will be carried out as needed following the completion of the work | Not significant |
| Environmental conditions | Construction and excavation | Alteration of soil quality | - Excavated soil will be segregated and stored in a manner that prevents leaching and wind dispersion. Surplus samples will be removed from the site and managed by the Contractor on the basis of a sampling method, in accordance with applicable regulations . | Not significant |
| Environmental conditions | Excavation | Effects of noise, machinery, vibration and light pollution on migratory birds | - Work only during the day, not at night - Avoid artificial lighting, especially at night. - Addition of mitigation and monitoring measures to avoid harming migratory birds. - In the event that additional vegetation cutting is required: Avoid the nesting period of migratory birds in the area. The calendar of migratory bird nesting periods, the nesting period for the region in which the project is located, runs from late April to late August. | Not significant |
| Species at risk (Bank Swallow) | Deforestation and habitat changes | Adverse effects of environmental changes on migratory birds. | - The Proponent must regularly inspect the piles of sand or earth and other structures suitable for nesting of the species, if present inside or near the work areas. These inspections should be conducted by a competent person and should aim to detect any active burrows located near the work and to ensure that the structures remain inaccessible for nesting | Not likely |
| Species at risk (Common nighthawk) | Deforestation and habitat changes | Adverse effects of environmental changes on migratory birds. | - The proponent must inspect the work areas regularly to detect the presence of active nests on the ground. - The proponent must create a worker training and awareness program on the presence of Common Nighthawk nests in work areas and on the protection measures to be implemented in the event of discovery. | Not likely |
| Environmental conditions | A post could be positioned on the bank of a watercourse. An area of 2m2 of possible permanent encroachment. | Water quality: Increase in pH due to the addition of concrete in the watercourse to install the transition line. | In the event that a pole is installed in a watercourse for the transition line: - The proponent must ensure that the pole to be placed in the watercourse is made of natural material. If a concrete block is added to the stream to stabilize the post, the proponent must implement the measures outlined in the MELCCFP's Fact Sheet – Management of Concrete Mixer and Concrete Pump Truck Washing Water During the Deconstruction Period. | Not likely |